Given an FAS and its PN model, the number of parts in the system to be processed is u, the capacity of each resource type is C(r),  r  R, the number of transitions and places in the PN model of the FAS is |T| and |P|, respectively. Denote the HBS algorithm with beam width b as HBS(b), and the time complexity of HBS(b) for scheduling the FAS is O(ub|T|2[u|T|2 + |P|CR + b|T|]), where CR = max{C(r) | r  R}.

Analysis: The HBS(b) algorithm consists of L = jZuLj search steps, depending on the number of levels in the reachability graph of the considered FAS, where Lj is the length of the operation-transition path of part j, i.e., the number of transitions on the operation-transition path of part j. In search step i (i  [1, L]), the nodes reserved in level i-1 are explored, as shown in Line 3-19 of Algorithm 1.

In each search step, at most beam width nodes are explored and at most b|T| new nodes can be generated for the next level. For a new node in level i, i  (0, L], the time complexity for calculating its completion time and evaluation functions are O(i|T|) and O(|P|CR), respectively. So the complexity for exploring all generated nodes is O(Lb|T|[i|T| + |P|CR]).

In each search step, the detection process of nodes with the same marking (Line 13) has a time complexity of O(b2|T|2/2), and the time complexity of sorting all nodes in each level (Line 20) is b|T|logb|T|.

Since i  L  u|T|, there is O(Lb|T|[i|T| + |P|CR]) + O(Lb|T|logb|T|) + O(Lb2|T|2/2)  O(u|T|b|T|[u|T||T| + |P|CR + logb|T|]) + O(u|T| b2|T|2/2) = O(ub|T|2[u|T|2 + |P|CR + b|T|]). Thus, HBS(b) algorithm has time complexity O(ub|T|2[u|T|2 + |P|CR + b|T|]).